ICT Office (SWA/UHS/Confucius)

DATE: 24/11/2025

WI-FI COVERAGE MAPPING.

A site survey was conducted to assess and map the WI-FI network across student halls and key staff areas after multiple complaints from students regarding poor network coverage. The assessment covered Halls 1-15, 20 as well as the staff clinic, ICEC, Lab, Sickbay & Theatre, CCU and SWA HQ.

Findings

- Large trees within the compounds significantly obstructed signal penetration from the outdoor APs.

- Most complaints originated from students on the 4th floor of the building, where outdoor signals failed to reach effectively.

- Limited penetration of signals to inner rooms due to the building structure and wall density which reduces signal strength or makes it not even reach the interior of the rooms and corridors. i.e. Hall 10, 11, 12, 13.

- Several areas lacked adequate coverage due to insufficient AP placement i.e. hall 3, 12, 13, 14

Recommendation

- Install indoor APs in TV rooms to ease congestion and more complaints, as already proven effective in Hall 9 and Mamlaka unit.

- Additional APs on blind spot zone to strengthen signal reach and eliminate coverage gaps.

- Prune overgrown trees to enhance signal propagation

- Increase overall network bandwidth.

Summary of the affected areas and AP requirements

| Location | Existing Outdoor APs | No of Indoors APs | No of Switches | Additional Outdoor APs Need on the Blind spot areas |
| --- | --- | --- | --- | --- |
| Hall 1 | 3 | 0 | 1 | 0 |
| Hall 2 | 3 | 0 | 1 | 0 |
| Hall 3 | 2 | 0 | 1 | 1 |
| Hall 4 | 4 | 0 | 1 | 1 |
| Hall 5 | 5 | 0 | 1 | 1 |
| Hall 6 | 3 | 0 | 1 | 1 |
| Hall 7 | 3 | 0 | 1 | 1 |
| Hall 8 | 4 | 0 | 1 | 0 |
| Hall 9 | 6 | 0 | 1 | 0 |
| Hall 10 | 6 | 0 | 1 | 2 |
| Hall 11 | 6 | 0 | 1 | 2 |
| Hall 12 | 4 | 0 | 1 | 3 |
| Hall 13 | 3 | 0 | 1 | 4 |
| Hall 14 (Stella Awinja | 2 | 0 | 1 | 2 |
| Hall 15 | 2 | 0 | 1 | 2 |
| SWA HQ | 1 | 2 | 2 | 0 |
| Hall 20 (Box, Women’s hall) | 5 | 0 | 2 | 1 |
| Kitchen 1 | 0 | 1 | 1 | 0 |
| Mamlaka Kitchen | 0 | 2 | 1 | 0 |
| Mamlaka A | 7 | 1 | 1 | 0 |
| Mamlaka B | 8 | 1 | 1 | 0 |
| Student Clinic & CMO | 0 | 2 | 1 | 0 |
| Sickbay, Gender Desk theatre, Lab, Account | 0 | 1 | 1 | 2 |
| CCU/ Translation Building | 0 | 3 | 4 | 1 |
| Staff Clinic | 0 | 1 | 1 | 1 |
| Total | 78 | 14 | 30 | 25 |

Women’s Hall requires an indoor AP at Dinning to improve coverage for students’ residents.

All proposed installation points have been mapped using Google My Maps for precise deployment.

Here is the link: google.com/maps/d/edit?mid=1DQYYrBpMDaWxKPK2h1ZpKIM_y1s-B5w

Prepared by:

Kipiama Brian

ICT User Support.